AD30NA6L (R100)

Features

- Onboard Intel Alder Lake-N series processor;

- 1 x DDR5 SO-DIMM, total maximum 16GB;

- 6 x Intel i226-V (4 x Intel i226-V optional);

- 1 x M.2 2280 SSD, E-Key, B-Key;

- 1 x HDMI2.0, TPM2.0;

- DC 12-19V; Size: 120 x 120mm.

Product Specifications

| Processor system | Onboard Intel Alder Lake-N series processor, TDP 15W |
| --- | --- |
|  | EFI BIOS |
| Memory | 1 x DDR5 SO-DIMM, total maximum 16GB |
| Storage | 1 x M.2 M-Key 2280 (SATA3.0 protocol) |
|  | 1 x SATA3.0, 2Pin 5V power supply |
| Display | 1 x HDMI2.0, support 4K@60Hz |
| Board edge I/O interface | Power button, reset button, 3 x USB3.0, 1 x USB2.0 |
|  | DC JACK, 1 x HDMI, 1 x Console |
|  | 6 x Intel i226-V (4 x Intel i226-V optional; POE optional for LAN1/2/3/4, POE has a pop-up switch) |
| Expansion interface/function | External TPM2.0 optional, not available by default. The default is CPU built-in TPM2.0 |
|  | 1 x M.2 E-Key (PCIe+USB2.0 protocol, WIFI/BT module; no PCIe protocol when using 4LAN) |
|  | 1 x M.2 B -Key (USB3.0+USB2.0 protocol, 4G/5G module) |
|  | 2 x RS232 pin headers, pitch 2.0mm; 8-bit GPIO, 2x5Pin, pitch 2.0mm |
|  | 1 x USB2.0 pin header, 2x5Pin, pitch 2.0mm; F _Panel |
|  | 1 x 4Pin PWM CPU FAN, 1 x 4Pin PWM SYS FAN |
| Power supply | DC 12-19V, 60W or above |
| Work environment | Working temperature: -20℃ ~ +60℃; working humidity: 5% ~ 90% |
|  | Storage temperature: -40℃ ~ +85℃; Storage humidity: 5% ~ 90% |
| Operating system | Windows 10, Windows 11, Linux |
| Size | 120 x 120 mm |
| Net weight | Without radiator, about 130g; with radiator, about 250g |

AD30NA6L(R100)

Measurement                                                  Unit: mm

Ordering Information

| Model | Optional items | Description |
| --- | --- | --- |
| AD30NA6L​​ | Onboard Intel Alder Lake-N series processor | N100/N305 etc.​​ |
|  | network | 6Lan or 4Lan for option |
|  | POE optional | POE optional for LAN1/2/3/4 |
|  | TPM2.0 | External TPM2.0 optional |